SE4420

A. Machemer

Homework 1

Question 1.

L=1Ti=1nLiTi=1303*5+4*10+2*6+(3*)=13015+40+12+27=9430=3.13

Question 2.

Maintainer data compiled and shown in Table 1.

Table 1. Helicopter Maintenance Shift Summary

| Helicopter Maintenance Service Shift Summary |  |  |  |  |  |  |  | Queue Length |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Heli # | Inter-Arrival Time (hrs) | Service Time (hrs) | Arrival Time | Service Start | Service End | Wait Time | Idle Time | Start | Finish |
| 1 | 0 | 2.2 | 0 | 0 | 2.2 | 0 | 0 | 0 | 1 |
| 2 | 1.4 | 1.2 | 1.4 | 2.2 | 3.4 | 0.8 | 0 | 1 | 0 |
| 3 | 2.1 | 0.3 | 3.5 | 3.5 | 3.8 | 0 | 0.1 | 0 | 0 |
| 4 | 1.9 | 2.8 | 5.4 | 5.4 | 8.2 | 0 | 1.6 | 0 | 2 |
| 5 | 0.2 | 0.2 | 5.6 | 8.2 | 8.4 | 2.6 | 0 | 2 | 1 |
| 6 | 1.5 | 1.8 | 7.1 | 8.4 | 10.2 | 1.3 | 0 | 1 | 0 |

Minimum Queue Length = 0

Maximum Queue Length = 2

Total Idle Time: 0.1 + 1.6 = 1.7 hours

Total Wait Time: 0.8+2.6+1.3 = 4.7 hours

Average Wait Time: W=1ni=1nWi=160+0.8+0+0+2.6+1.3=4.76=0.783 hours

Utilization:

Total Time on Shift = 10.2 hours

Total Idle Time = 1.7 hours

un=(10.2-1.7)10.2=0.833= 83.3%

Question 3.

Data compiled and shown in Table 2.

Table 2.  Shipyard Service Summary

| Ship | Interarrival Time | Service Time | Arrival Time | Service Start | Service End | Wait Time | Idle Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 2.5 | 0 | 0 | 2.5 | 0 | 0 |
| 3 | 1 | 2 | 2 | 2.5 | 4.5 | 0.5 | 0 |
| 5 | 1 | 3 | 4 | 4.5 | 7.5 | 0.5 | 0 |
| 6 | 1 | 1.5 | 5 | 7.5 | 9 | 2.5 | 0 |
| 8 | 1 | 2 | 7 | 9 | 11 | 2 | 0 |
| 9 | 1 | 1 | 8 | 11 | 12 | 3 | 0 |
| 13 | 1 | 2 | 12 | 12 | 14 | 0 | 0 |
| 16 | 1 | 0.5 | 15 | 15 | 15.5 | 0 | 1 |
| 18 | 1 | 1.5 | 17 | 17 | 18.5 | 0 | 1.5 |
| 19 | 1 | 3 | 18 | 18.5 | 21.5 | 0.5 | 0 |

Note: Table curtailed to only ships being serviced.

Average Wait Time: W=1ni=1nWi=1100.5+0.5+2.5+2+3+0+0+0+0.5=910=0.9 days

Utilization: un=0T(n)B(t)T(n)=12013.5-0*1+14.5-13.5*0+15.5-14.5*1+17-15.5*0+20-17*1=13.5+1+320=17.520=0.875=87.5%

Numbers derived from analyzing Shipyard Utilization plot (fig 1).

Fig. 1: Shipyard Utilization as a Function

Question 4: C, it will decrease. The total server capacity has doubled in this model and, assuming the arrival rate does not change and the service is allocated to whichever server is available, this decreases the amount of work per server and thus decreases the utilization per server.  However, it does so without decreasing the amount of work cumulatively completed by both, but with decreased wait times and queue lengths.